CHAPTER 3

CITY PAIRS & ROUTES

Table Of Content

3.1 GENERAL

3.2 POLICY

3.3 STANDARD ROUTES AND RESTRICTIONS

3.4 POSITION FIXING, REPORTS AND WAYPOINTS

3.5 ENROUTE CHARTS

# 3.1 GENERAL

The purpose of this chapter is to provide supplemental information pertaining to domestic and international routes operated by Vietnam Airlines.

# 3.2 POLICY

All routes operated by VNA will be up-to-date and appropriate for the proposed types and areas of operations.

A route guide to ensure that the flight crew will have, for each flight, information relating to communication facilities, navigation aids, aerodromes, instrument approaches, instruments arrivals and instruments departures as applicable for the operation, and such other information that may deem necessary for the proper conduct of flight operations – Refer VAR 12.205, VAR 12.190, FOM 1.5.2

Vietnam Airlines will provide individual flight plans and airway routing descriptors to the flight crews for each flight. These flight plans shall reflect the standard routes outlined in section 3.3. In the event of any discrepancy in routing between flight plans and database or information in this manual, the flight crews shall comply with the information provided in the flight plans.

# 3.3 STANDARD ROUTES AND RESTRICTIONS

## 3.3.1 GENERAL INFORMATION

City pairs, RVSM/PBN routes, and flight levels for each route will be listed in section 3.3.2.

2

1

a) Decode Header:

- SGN – SFO – SGN

- Number ranked in the table

- Name of city pairs in IATA code

b) Header Guide:

Table 3-1: Header Description

| Destination Area Operation | Number rank | Header Color |
| --- | --- | --- |
| North American | 1 |  |
| Europe | 2-13 |  |
| Australia | 14-19 |  |
| Mid-East | 20-29 |  |
| Northeast Asia | 30-82 |  |
| Southeast Asia | 83-99 |  |
| Vietnam (Domestic) | 100-149 |  |

## 3.3.2 TABLE OF CITY PAIRS AND STANDARD ROUTES

3.3.2.1 NORTH AMERICAN

| No | City pairs | Standard routes DB name | Preferred destination alternate airports |
| --- | --- | --- | --- |

| - SGN – SFO – SGN |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | VVTS | KSFO | SGNSFO25 | KOAK, KSJC, KLAX |
|  | KSFO | VVTS | RUS01 | VVCT, VVCR |
| Area Operation | MID/ASIA, PAC, NAM |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, RPLL, RCTP, ROAH, RJBB, RJTT, RJCC, PACD, CYVR |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 / 207/ 240 |  |  |  |
| PBN | RNAV2, RNAV 5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, PBCS, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | All IFR Flight levels at or above FL290 except the westbound North American – Japan PACOTS which also includes FL280 in the Oakland OCA/FIR. |  |  |  |

3.3.2.2 EUROPE

| No | City pairs | Standard routes DB name | Preferred destination alternate airports |
| --- | --- | --- | --- |

| - HAN – AMS – HAN |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | VVNB | EHAM | HANAMS01 | EDDL, EBBR, EDDF |
|  | EHAM | VVNB | AMSHAN01 | VVCI, VVDN |
| Area Operation | MID/ASIA, EUR |  |  |  |
| Preferred Enroute Alternate Airport | VTCC, VYYY, VECC, VIDP, VIAR, UTDD, UZSA, UBBB, UBBG, LTCE, LTFM, LHBP, EDDB |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Depressurization Procedure | YES (refer to Chapter 9 Escape Route Procedures - Route Manual) |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| - HAN – CDG – HAN |  |  |  |  |
|  | VVNB | LFPG | HANCDGP500 | LFPO, EBBR, EGLL |
|  | LFPG | VVNB | CDGHANP500 | VVCI, VVDN |
| Area Operation | MID/ASIA, EUR |  |  |  |
| Preferred Enroute Alternate Airport | VTCC, VYYY, VECC, VIDP, VIAR, UTDD, UZSA, UBBB, LTCE, LROP |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Depressurization Procedure | YES (refer to Chapter 9 Escape Route Procedures - Route Manual) |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway M890: Flights shall flight plan at and above FL300 |  |  |  |
| - HAN – FRA – HAN |  |  |  |  |
|  | VVNB | EDDF | HANFRAP500 | EDDL, EDDK, LFPG |
|  | EDDF | VVNB | FRAHANP500 | VVCI, VVVH |
| Area Operation | MID/ASIA, EUR |  |  |  |
| Preferred Enroute Alternate Airport | VTCC, VYYY, VECC, VIDP, VIAR, UTDD, UZSA, UBBB, LTCE, LROP |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Depressurization Procedure | YES (refer to Chapter 9 Escape Route Procedures - Route Manual) |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway M890: Flights shall flight plan at and above FL300 |  |  |  |
| - HAN – KZN – HAN |  |  |  |  |
|  | VVNB | UWKD | HANKZN | UUEE, USSS |
|  | UWKD | VVNB | KZNHAN | VVCI, VVVH |
| Area Operation | MID/ASIA, EUR |  |  |  |
| Preferred Enroute Alternate Airport | ZPPP, ZLXY, ZMCK, UAAA, UNNT, USSS |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Depressurization Procedure | YES (refer to Chapter 9 Escape Route Procedures - Route Manual) |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – LED – HAN |  |  |  |  |
| MID/ASIA | VVNB | ULLI | HANLED | UUWW, UUEE |
|  | ULLI | VVNB | LEDHAN | VVCI, VVVH |
| Area Operation | MID/ASIA, EUR |  |  |  |
| Preferred Enroute Alternate Airport | ZPPP, ZLXY, ZMCK, UAAA, UNNT, USSS, UUEE |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Depressurization Procedure | YES (refer to Chapter 9 Escape Route Procedures - Route Manual) |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – LHR – HAN |  |  |  |  |
|  | VVNB | EGLL | HANLHRP500 | EGKK, EGNX |
|  | EGLL | VVNB | LHRHANP500 | VVCI, VVVH |
| Area Operation | MID/ASIA, EUR |  |  |  |
| Preferred Enroute Alternate Airport | VTCC, VYYY, VECC, VIDP, VIAR, UTDD, UZSA, UBBB, LTCE, LROP |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Depressurization Procedure | YES (refer to Chapter 9 Escape Route Procedures - Route Manual) |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway M890: Flights shall flight plan at and above FL300 |  |  |  |
| - HAN – MUC – HAN |  |  |  |  |
| MID/ASIA | VVNB | EDDM | HANMUCP500 | EDDF, EDDS, LFPG |
|  | EDDM | VVNB | MUCHANP500 | VVCI, VVVH |
| Area Operation | MID/ASIA, EUR |  |  |  |
| Preferred Enroute Alternate Airport | VTCC, VYYY, VECC, VIDP, VIAR, UTDD, UZSA, UBBB, LTCE, LROP |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Depressurization Procedure | YES (refer to Chapter 9 Escape Route Procedures - Route Manual) |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway M890: Flights shall flight plan at and above FL300 |  |  |  |
| - HAN – MXP – HAN |  |  |  |  |
|  | VVNB | LIMC | HANMXP | LIRF, EDDF, LFPG |
|  | LIMC | VVNB | MXPHAN | VVCI, VVVH |
| Area Operation | MID/ASIA, EUR |  |  |  |
| Preferred Enroute Alternate Airport | VTCC, VYYY, VECC, VIDP, VIAR, UTDD, UZSA, UBBB, LTCE, LROP |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Depressurization Procedure | YES (refer to Chapter 9 Escape Route Procedures - Route Manual) |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway M890: Flights shall flight plan at and above FL300 |  |  |  |
| - HAN – SVO – HAN |  |  |  |  |
|  | VVNB | UUEE | HANSVO | UUDD, UUWW, UWKD |
|  | UUEE | VVNB | SVOHAN | VVCI, VVVH |
| Area Operation | MID/ASIA, EUR |  |  |  |
| Preferred Enroute Alternate Airport | VTCC, VYYY, VECC, VIDP, VIAR, UTDD, UZSA, UBBB, LTCE, LROP |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Depressurization Procedure | YES (refer to Chapter 9 Escape Route Procedures - Route Manual) |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – CDG – SGN |  |  |  |  |
|  | VVTS | LFPG | SGNCDGP500 | LFPO, EBBR, EGLL |
|  | LFPG | VVTS | CDGSGNP500 | VVCT, VVCR |
| Area Operation | MID/ASIA, EUR |  |  |  |
| Preferred Enroute Alternate Airport | VYYY, VECC, VIDP, VIAR, UTDD, UZSA, UBBB, LTCE, LROP |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Depressurization Procedure | YES (refer to Chapter 9 Escape Route Procedures - Route Manual) |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway M890: Flights shall flight plan at and above FL300 |  |  |  |
| - SGN – FRA – SGN |  |  |  |  |
|  | VVTS | EDDF | SGNFRAP500 | EDDL, EDDK, LFPG |
|  | EDDF | VVTS | FRASGNP500 | VVCT, VVCR |
| Area Operation | MID/ASIA, EUR |  |  |  |
| Preferred Enroute Alternate Airport | VYYY, VECC, VIDP, VIAR, UTDD, UZSA, UBBB, LTCE, LROP |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Depressurization Procedure | YES (refer to Chapter 9 Escape Route Procedures - Route Manual) |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway M890: Flights shall flight plan at and above FL300 |  |  |  |
| - SGN – LHR – SGN |  |  |  |  |
|  | VVTS | EGLL | SGNLHRP500 | EGKK, EGNX |
|  | EGLL | VVTS | LHRSGNP500 | VVCT, VVCR |
| Area Operation | MID/ASIA, EUR |  |  |  |
| Preferred Enroute Alternate Airport | VYYY, VECC, VIDP, VIAR, UTDD, UZSA, UBBB, LTCE, LROP |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Depressurization Procedure | YES (refer to Chapter 9 Escape Route Procedures - Route Manual) |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway M890: Flights shall flight plan at and above FL300 |  |  |  |
| - SGN – MUC – SGN |  |  |  |  |
|  | VVTS | EDDM | SGNMUCP500 | EDDB, ESSA, ENGM |
|  | EDDM | VVTS | MUCSGNP500 | VVCT, VVCR, VVDN |
| Area Operation | MID/ASIA, EUR |  |  |  |
| Preferred Enroute Alternate Airport | VYYY, VECC, VIDP, VIAR, UTDD, UZSA, UBBB, LTCE, LROP |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Depressurization Procedure | YES (refer to Chapter 9 Escape Route Procedures - Route Manual) |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway M890: Flights shall flight plan at and above FL300 |  |  |  |
| - SGN – CPH – SGN |  |  |  |  |
|  | VVTS | EKCH | SGNCPHL750 | EDDF, EDDS, LFPG |
|  | EKCH | VVTS | CPHSGNL750 | VVCT, VVCR |
| Area Operation | MID/ASIA, EUR |  |  |  |
| Preferred Enroute Alternate Airport | VLVT, VYYY, VECC, VIDP, VIAR, UZSA, UBBG, LTCE, LROP, LOWW, EDDB |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Depressurization Procedure | YES (refer to Chapter 9 Escape Route Procedures - Route Manual) |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway M890: Flights shall flight plan at and above FL300 |  |  |  |
| - SGN – SVO – SGN |  |  |  |  |
|  | VVTS | UUEE | SGNSVO | UUDD, UUWW, UWKD |
|  | UUEE | VVTS | SVOSGN | VVCI, VVVH |
| Area Operation | MID/ASIA, EUR |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VVNB, VTCC, VYYY, VECC, VIDP, VIAR, UTDD, UZSA, UBBB, LTCE, LROP |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Depressurization Procedure | YES (refer to Chapter 9 Escape Route Procedures - Route Manual) |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |

3.3.2.3 AUSTRALIA

| No | City pairs | Standard routes DB name | Preferred destination alternate airports |
| --- | --- | --- | --- |

| - HAN – MEL – HAN |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | VVNB | YMML | HANMEL1134A | YSCB, YSSY, YPAD |
|  | YMML | VVNB | MELHAN1611 | VVCI, VVDN |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVTS, WBKK, WAAA, YPDN, YBAS, YPAD |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV2, RNAV 5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, PBCS, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – PER – HAN |  |  |  |  |
|  | VVNB | YPPH | HANPER | YGEL, YMML |
|  | YPPH | VVNB | PERHAN | VVCI, VVDN |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVTS, WBSB, WADD, WSSS, WIII, YPKA, YGEL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV 5, RNP4 (RNAV4), RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, PBCS, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – SYD – HAN |  |  |  |  |
|  | VVNB | YSSY | HANSYD1116A | YBBN, YSCB, YMML |
|  | YSSY | VVNB | SYDHAN1611 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVTS, WBKK, WAAA, YPDN, YBAS, YBBN |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, PBCS, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – MEL – SGN |  |  |  |  |
|  | VVTS | YMML | SGNMEL2516A | YSCB, YSSY, YPAD |
|  | YMML | VVTS | MELSGN1625 | VVCT, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | WBKK, WAAA, YPDN, YBAS, YPAD |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV2, RNAV 5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, PBCS, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – PER – SGN |  |  |  |  |
|  | VVTS | YPPH | SGNPER2521A | YGEL, YMML |
|  | YPPH | VVTS | PERSGN0325B | VVCT, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | WBSB, WADD, WSSS, WIII, YPKA, YGEL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNP4 (RNAV4), RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, PBCS, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – SYD – SGN |  |  |  |  |
|  | VVTS | YSSY | SGNSYD2516A | YBBN, YSCB, YMML |
|  | YSSY | VVTS | SYDSGN1625 | VVCT, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | WBKK, WAAA, YPDN, YBAS, YBBN |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | If required 120 / 180 |  |  |  |
| PBN | RNAV5, RNP4 (RNAV4), RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, PBCS, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |

3.3.2.4 MID-EAST

| No | City pairs | Standard routes DB name | Preferred destination alternate airports |
| --- | --- | --- | --- |

| - HAN – BOM – HAN |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | VVNB | VABB | HANBOM1109 | VAAH, VOHS |
|  | VABB | VVNB | BOMHAN0911 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VTCC, VECC, VOHS |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway M890: Flights shall flight plan at and above FL300 |  |  |  |
| - HAN – DEL – HAN |  |  |  |  |
|  | VVNB | VIDP | HANDEL1111 | VIJP, VIAR, VAAH |
|  | VIDP | VVNB | DELHAN2711 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VTCC, VECC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV10 (RNP 10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway M890: Flights shall flight plan at and above FL300 |  |  |  |
| - HAN – GAY – HAN |  |  |  |  |
|  | VVNB | VEGY | HANGAY1128 | VECC, VGHS, VILK |
|  | VEGY | VVNB | GAYHAN1011 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VTCC, VECC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway M890: Flights shall flight plan at and above FL300 |  |  |  |
| - HAN – VNS – HAN |  |  |  |  |
|  | VVNB | VEBN | HANVNS1109 | VEGY, VIDP |
|  | VEBN | VVNB | VNSHAN2711 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VECC, VYMD |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway M890: Flights shall flight plan at and above FL300 |  |  |  |
| - SGN – BOM – SGN |  |  |  |  |
|  | VVTS | VABB | SGNBOM2509 | VAAH, VOHS |
|  | VABB | VVTS | BOMSGN0925 | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VTBS, VYYY, VEBS, VOHS |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway M890: Flights shall flight plan at and above FL300 |  |  |  |
| - SGN – DEL – SGN |  |  |  |  |
|  | VVTS | VIDP | SGNDEL2511A | VIJP, VIAR, VAAH |
|  | VIDP | VVTS | DELSGN2725A | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VTCC, VECC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV10 (RNP 10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway M890: Flights shall flight plan at and above FL300 |  |  |  |
| - SGN – GAY – SGN |  |  |  |  |
|  | VVTS | VEGY | SGNGAY2528 | VECC, VGHS, VILK |
|  | VEGY | VVTS | GAYSGN1025 | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VTCC, VECC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway M890: Flights shall flight plan at and above FL300 |  |  |  |
| - HAN – KTM – HAN |  |  |  |  |
|  | VVNB | VNKT | HANKMT | VILK, VIDP |
|  | VNKT | VVNB | KMTHAN | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VTCC, VECC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – BLR – HAN |  |  |  |  |
|  | VVNB | VOBL | HANBLR | VOHS, VOMM, VABB |
|  | VOBL | VVNB | BLRHAN | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VTCC, VECC, VOHS |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – HYD – HAN |  |  |  |  |
|  | VVNB | VOHS | HANHYD | VOHS, VOMM, VABB |
|  | VOHS | VVNB | HYDHAN | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VTCC, VECC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – CMB – SGN |  |  |  |  |
|  | VVTS | VCBI | SGNCMB01 | VORV, VOBL, VOMM |
|  | VCBI | VVTS | CMBSGN01 | VVCR, VVPQ, VVCA |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VTCC, VECC, VOHS |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – JAI – HAN |  |  |  |  |
|  | VVNB | VIJP | HANJAI01 | VIDP, VAAH, VABB |
|  | VIJP | VVNB | JAIHAN01 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VECC, VYMD, VEBN |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |

| - HAN – AMD – HAN |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | VVNB | VAAH | HANAMD01 | VAID, VIJP, VABB, VABO |
|  | VAAH | VVNB | AMDHAN01 | VVCI, VVVH, VVDN |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VTCC, VLVT, VYYY, VYMD, VGHS, VECC, VEBS, VOHS, VIDP, VIJP |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |

3.3.2.5 NORTHEAST ASIA

| No | City pairs | Standard routes DB name | Preferred destination alternate airports |
| --- | --- | --- | --- |

| - CXR – CJU – CXR |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | VVCR | RKPC | CXRCJU0207A | RKSI, RKPK |
|  | RKPC | VVCR | CJUCXR0702 | VVTS, VVBM |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH, RCTP, RKSI |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL300, FL340; FL400 (W-BOUND) |  |  |  |

| - CXR – ICN – CXR |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | VVCR | RKSI | CXRICN0234A | RKSS, RKPK, RKTU |
|  | RKSI | VVCR | ICNCXR3402B | VVTS, VVBM |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH, RCTP, RKPC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL300, FL340; FL400 (W-BOUND) |  |  |  |
| - CXR – KIX – CXR |  |  |  |  |
|  | VVCR | RJBB | CXRKIX | RJOO, RJGG, RJFF |
|  | RJBB | VVCR | KIXCXR | VVTS, VVBM |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH, RCTP, ROAH |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL300, FL340; FL400 (W-BOUND) |  |  |  |
| - CXR – MWX – CXR |  |  |  |  |
| ASIA | VVCR | RKJB | CXRMWX0201 | RKSI, RKPK |
|  | RKJB | VVCR | MWXCXR0102 | VVTS, VVBM |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH, RCTP |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL300, FL340; FL400 (W-BOUND) |  |  |  |
| - CXR – NRT – CXR |  |  |  |  |
|  | VVCR | RJAA | CXRNRT | RJTT, RJGG |
|  | RJAA | VVCR | NRTCXR | VVTS, VVBM |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH, RCTP, ROAH, RJBB |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL300, FL340; FL400 (W-BOUND) |  |  |  |
| - DAD – AXT – DAD |  |  |  |  |
|  | VVDN | RKTU | DADAXT3528A | RJAA, RJGG |
|  | RKTU | VVDN | AXTDAD | VVPB, VVCR, VVTS |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VHHH, RCTP, ROAH, RJBB |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL300, FL340; FL400 (W-BOUND) Airway A593: THE EAST OF SADLI: Flights shall flight plan FL250, FL290, FL310 or FL390 (E-BOUND); FL240 FL280, FL300 or FL400 (W-BOUND) |  |  |  |
| - DAD – CJJ – DAD |  |  |  |  |
|  | VVDN | RKTU | DADCJJ | RKSI, RKPK |
|  | RKTU | VVDN | CJJDAD | VVPB, VVCR, VVTS |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VHHH, RCTP, RKPC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL300, FL340; FL400 (W-BOUND) Airway A593: THE EAST OF SADLI: Flights shall flight plan FL250, FL290, FL310 or FL390 (E-BOUND); FL240 FL280, FL300 or FL400 (W-BOUND)) |  |  |  |
| - DAD – CJU – DAD |  |  |  |  |
|  | VVDN | RKPC | DADCJU | RKSI, RKPK |
|  | RKPC | VVDN | CJUDAD | VVPB, VVCR, VVTS |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute alternate Airport | VVDN, VHHH, RCTP, RKSI |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL300, FL340; FL400 (W-BOUND) Airway A593: THE EAST OF SADLI: Flights shall flight plan FL250, FL290, FL310 or FL390 (E-BOUND); FL240 FL280, FL300 or FL400 (W-BOUND) |  |  |  |
| - DAD – FUK – DAD |  |  |  |  |
|  | VVDN | RJFF | DADFUK | RKPK, RJFU, RJBB |
|  | RJFF | VVDN | FUKDAD | VVPB, VVCR, VVTS |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VHHH, RCTP |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL300, FL340, FL400 (W-BOUND); FL290, FL330, FL370, FL410 (E-BOUND) and shall be restricted to flights transiting from Taipei FIR to Fukuoka FIR via IGURU between 1700-2200 UTC (E-BOUND) |  |  |  |
| - DAD – ICN – DAD |  |  |  |  |
|  | VVDN | RKSI | DADICN3536A | RKSS, RKPK, RKTU |
|  | RKSI | VVDN | ICNDAD3435A | VVPB, VVCR, VVTS |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VHHH, RCTP, RKPC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL300, FL340; FL400 (W-BOUND) Airway A593: THE EAST OF SADLI: Flights shall flight plan FL250, FL290, FL310 or FL390 (E-BOUND); FL240 FL280, FL300 or FL400 (W-BOUND) |  |  |  |
| - DAD – KIX – DAD |  |  |  |  |
|  | VVDN | RJBB | DADKIX3506A | RJOO, RJGG, RJFF |
|  | RJBB | VVDN | KIXDAD2435 | VVPB, VVCR, VVTS |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VHHH, RCTP, ROAH |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL290, FL330, FL370, FL410; shall be restricted to flights transiting from Taipei FIR to Fukuoka FIR via IGURU between 1700-2200 UTC (E-BOUND) |  |  |  |
| - DAD – MWX – DAD |  |  |  |  |
|  | VVDN | RKJB | DADMWX3501 | RKSI, RKPK |
|  | RKJB | VVDN | MWXDAD0135 | VVPB, VVCR, VVTS |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VHHH, RCTP |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL300, FL340; FL400 (W-BOUND) Airway A593: THE EAST OF SADLI: Flights shall flight plan FL250, FL290, FL310 or FL390 (E-BOUND); FL240 FL280, FL300 or FL400 (W-BOUND) |  |  |  |
| - DAD – NGO – DAD |  |  |  |  |
|  | VVDN | RJGG | DADNGO | RJBB, RJTT, RJAA |
|  | RJGG | VVDN | NGODAD | VVPB, VVCR, VVTS |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VHHH, RCTP, ROAH |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL300, FL340, FL400 (W-BOUND); FL290, FL330, FL370, FL410 (E-BOUND) and shall be restricted to flights transiting from Taipei FIR to Fukuoka FIR via IGURU between 1700-2200 UTC (E-BOUND) |  |  |  |
| - DAD – NRT – DAD |  |  |  |  |
|  | VVDN | RJAA | DADNRT3534A | RJTT, RJGG, RJBB |
|  | RJAA | VVDN | NRTDAD1635A | VVPB, VVCR, VVTS |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VHHH, RCTP, ROAH, RJBB |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL300, FL340, FL400 (W-BOUND); FL290, FL330, FL370, FL410 (E-BOUND) and shall be restricted to flights transiting from Taipei FIR to Fukuoka FIR via IGURU between 1700-2200 UTC (E-BOUND) |  |  |  |
| - DAD – PUS – DAD |  |  |  |  |
|  | VVDN | RKPK | DADPUS3536A | RKPC, RJFF, RKPK |
|  | RKPK | VVDN | PUSDAD3635 | VVPB, VVCR, VVTS |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VHHH, RCTP, RKPC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – FUK – HAN |  |  |  |  |
|  | VVNB | RJFF | HANFUK1116 | RKPK, RJFU, RJBB |
|  | RJFF | VVNB | FUKHAN1611A | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | ZGGG, ZHCC, ZSPD |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway A593: THE EAST OF SADLI: Flights shall flight plan FL250, FL290, FL310 or FL390 (E-BOUND); FL240 FL280, FL300 or FL400 (W-BOUND) |  |  |  |
| - HAN – HND – HAN |  |  |  |  |
|  | VVNB | RJTT | HANHND1116 | RJAA, RJGG, RJBB |
|  | RJTT | VVNB | HNDHAN1611A | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | ZGGG, ZSPD, RKPC, RJFF |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway A593: THE EAST OF SADLI: Flights shall flight plan FL250, FL290, FL310 or FL390 (E-BOUND); FL240 FL280, FL300 or FL400 (W-BOUND) |  |  |  |
| - HAN – ICN – HAN |  |  |  |  |
|  | VVNB | RKSI | HANICNR343 | RKSS, RKPK, RKTU |
|  | RKSI | VVNB | ICNHAN1511A | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | ZGGG, ZSPD, VHHH |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway A593: THE EAST OF SADLI: Flights shall flight plan FL250, FL290, FL310 or FL390 (E-BOUND); FL240 FL280, FL300 or FL400 (W-BOUND) |  |  |  |
| - HAN – IZO – HAN |  |  |  |  |
|  | VVNB | RJOC | HANIZO1107 | RJBB, RJAA |
|  | RJOC | VVNB | IZOHAN0711 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | ZGGG, ZSPD, RJFF, RJBB, RKPC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 REMARKS: NIL |  |  |  |
| Restriction | Airway A593: THE EAST OF SADLI: Flights shall flight plan FL250, FL290, FL310 or FL390 (E-BOUND); FL240 FL280, FL300 or FL400 (W-BOUND) |  |  |  |
| - HAN – KHH – HAN |  |  |  |  |
|  | VVNB | RCKH | HANKHH1109A | RCTP, VHHH |
|  | RCKH | VVNB | KHHHAN0911A | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VHHH, ZGNN |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – KIX – HAN |  |  |  |  |
|  | VVNB | RJBB | HANKIXR343 | RJOO, RJGG, RJFF |
|  | RJBB | VVNB | KIXHAN2411A | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | ZGGG, ZSPD, RJFF, RKPC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway A593: THE EAST OF SADLI: Flights shall flight plan FL250, FL290, FL310 or FL390 (E-BOUND); FL240 FL280, FL300 or FL400 (W-BOUND) |  |  |  |
| - HAN – KOJ – HAN |  |  |  |  |
|  | VVNB | RJFK | HANKOJ1134 | RJFT, RJFF, RJFK |
|  | RJFK | VVNB | KOJHAN1611 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | ZGGG, ZHCC, ZSPD |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway A593: THE EAST OF SADLI: Flights shall flight plan FL250, FL290, FL310 or FL390 (E-BOUND); FL240 FL280, FL300 or FL400 (W-BOUND) |  |  |  |
| - HAN – NGO – HAN |  |  |  |  |
|  | VVNB | RJGG | HANNGOR343 | RJBB, RJTT, RJAA |
|  | RJGG | VVNB | NGOHAN1811A | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | ZGGG, ZHCC, ZSPD, RJFF |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway A593: THE EAST OF SADLI: Flights shall flight plan FL250, FL290, FL310 or FL390 (E-BOUND); FL240 FL280, FL300 or FL400 (W-BOUND) |  |  |  |
| - HAN – NRT – HAN |  |  |  |  |
|  | VVNB | RJAA | HANNRTR343 | RJTT, RJGG, RJBB |
|  | RJAA | VVNB | NRTHAN1611A | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | ZGGG, ZHCC, ZSPD, RJBB, RKPC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway A593: THE EAST OF SADLI: Flights shall flight plan FL250, FL290, FL310 or FL390 (E-BOUND); FL240 FL280, FL300 or FL400 (W-BOUND) |  |  |  |
| - HAN – OKA – HAN |  |  |  |  |
|  | VVNB | ROAH | HANOKA | RCTP, RJFK |
|  | ROAH | VVNB | OKAHAN | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | ZGGG, ZHCC, ZSPD, RCTP, RCKH |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – PUS – HAN |  |  |  |  |
|  | VVNB | RKPK | HANPUSR343 | RKPC, RJFF, RKPK |
|  | RKPK | VVNB | PUSHAN3611 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | ZGGG, ZHCC, ZSPD, VHHH |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway A593: THE EAST OF SADLI: Flights shall flight plan FL250, FL290, FL310 or FL390 (E-BOUND); FL240 FL280, FL300 or FL400 (W-BOUND) |  |  |  |
| - HAN – TPE – HAN |  |  |  |  |
|  | VVNB | RCTP | HANTPE1105 | RCHK, VHHH |
|  | RCTP | VVNB | TPEHAN0511 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | ZGNN, VHHH |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – CJJ – SGN |  |  |  |  |
|  | VVTS | RKTU | SGNCJJ | RKSI, RKPK |
|  | RKTU | VVTS | CJJSGN | VVCA, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH, RCTP, RKPC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL300, FL340, FL400 (W-BOUND) Airway A593: THE EAST OF SADLI: Flights shall flight plan FL250, FL290, FL310 or FL390 (E-BOUND); FL240 FL280, FL300 or FL400 (W-BOUND) |  |  |  |
| - SGN – CJU – SGN |  |  |  |  |
|  | VVTS | RKPC | SGNCJU | RKSI, RKPK |
|  | RKPC | VVTS | CJUSGN | VVCA, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH, RCTP, RKSI |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – FUK – SGN |  |  |  |  |
|  | VVTS | RJFF | SGNFUK2516A | RKPK, RJFU, RJBB |
|  | RJFF | VVTS | FUKSGN1625 | VVCA, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVCR, VHHH, RCTP, ROAH |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL290, FL330, FL370, FL410; shall be restricted to flights transiting from Taipei FIR to Fukuoka FIR via IGURU between 1700-2200 UTC (E-BOUND) |  |  |  |
| - SGN – ICN – SGN |  |  |  |  |
|  | VVTS | RKSI | SGNICN2515 | RKSS, RKPK, RKTU |
|  | RKSI | VVTS | ICNSGN1525 | VVCA, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH, RCTP, RKPC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL300, FL340, FL400 (W-BOUND) Airway A593: THE EAST OF SADLI: Flights shall flight plan FL250, FL290, FL310 or FL390 (E-BOUND); FL240 FL280, FL300 or FL400 (W-BOUND) |  |  |  |
| - SGN – KIX – SGN |  |  |  |  |
|  | VVTS | RJBB | SGNKIX2506A | RJOO, RJGG, RJFF |
|  | RJBB | VVTS | KIXSGN0625A | VVCA, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH, RCTP, ROAH |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL290, FL330, FL370, FL410; shall be restricted to flights transiting from Taipei FIR to Fukuoka FIR via IGURU between 1700-2200 UTC (E-BOUND) |  |  |  |
| - SGN – MWX – SGN |  |  |  |  |
|  | VVTS | RKJB | SGNMWX | RKSI, RKPK |
|  | RKJB | VVTS | MWXSGN | VVCA, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH, RCTP |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL300, FL340, FL400 (W-BOUND) Airway A593: THE EAST OF SADLI: Flights shall flight plan FL250, FL290, FL310 or FL390 (E-BOUND); FL240 FL280, FL300 or FL400 (W-BOUND) |  |  |  |
| - SGN – NGO – SGN |  |  |  |  |
|  | VVTS | RJGG | SGNNGO2536A | RJBB, RJTT, RJAA |
|  | RJGG | VVTS | NGOSGN3625 | VVCA, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH, RCTP, ROAH |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL290, FL330, FL370, FL410; shall be restricted to flights transiting from Taipei FIR to Fukuoka FIR via IGURU between 1700-2200 UTC (E-BOUND) |  |  |  |
| - SGN – NRT – SGN |  |  |  |  |
|  | VVTS | RJAA | SGNNRT2516 | RJTT, RJGG, RJBB |
|  | RJAA | VVTS | NRTSGN1625 | VVCA, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH, RCTP, ROAH, RJBB |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL290, FL330, FL370, FL410; shall be restricted to flights transiting from Taipei FIR to Fukuoka FIR via IGURU between 1700-2200 UTC (E-BOUND) |  |  |  |
| - SGN – PUS – SGN |  |  |  |  |
|  | VVTS | RKPK | SGNPUS2536A | RKPC, RJFF, RKPK |
|  | RKPK | VVTS | PUSSGN3625A | VVCA, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH, RCTP, RKPC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL300, FL340, FL400 (W-BOUND) Airway A593: THE EAST OF SADLI: Flights shall flight plan FL250, FL290, FL310 or FL390 (E-BOUND); FL240 FL280, FL300 or FL400 (W-BOUND) |  |  |  |
| - SGN – TPE – SGN |  |  |  |  |
| MID/ASIA | VVTS | RCTP | SGNTPE2505A | RCHK, VHHH |
|  | RCTP | VVTS | TPESGN0525 | VVCA, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – HNA – TPE – DAD |  |  |  |  |
|  | VVNB | RJSI | HANHNA1102 | RJAA, RJGG |
|  | RJSI | RCTP | HNATPE0105 | RCHK, VHHH |
|  | RCTP | VVDN | TPEDAD2335 | VVPB, VVVH, VVNB |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | ZGNN, ZGGG, VHHH, RCKH, RJFF, RJBB |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - DAD – HNA – AXT – TPE – DAD |  |  |  |  |
|  | VVDN | RJSI | DADHNA | RJAA, RJGG |
|  | RJSI | RJSK | HNAAXT | RJAA, RJGG |
|  | RJSK | RCTP | AXTTPE | RCHK, VHHH |
|  | RCTP | VVDN | TPEDAD | VVPB, VVVH, VVNB |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | ZGNN, ZGGG, VHHH, RCKH, RJFF, RJBB |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |

| - CXR – CGO – CXR |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | VVCR | ZHCC | CXRCGO0212A | ZBAA, ZBYN, ZHHH |
|  | ZHCC | VVCR | CGOCXR1202 | VVTS, VVBM |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNP 10 (RNAV 10) |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: NIL |  |  |  |
| Restriction | NIL |  |  |  |

| - CXR – SHE – CXR |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | VVCR | ZYTX | CXRSHE0206A | ZYTL, ZBAA |
|  | ZYTX | VVCR | SHECXR0602 | VVTS, VVBM |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH, ZGHA, ZBTJ |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNP 10 (RNAV 10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: NIL |  |  |  |
| Restriction | NIL |  |  |  |
| - CXR – SJW – CXR |  |  |  |  |
|  | VVCR | ZBSJ | CXRSJW0215 | ZBAA, ZBTJ |
|  | ZBSJ | VVCR | SJWCXR1502 | VVTS, VVBM |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVNB, VHHH, ZGHA |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: NIL |  |  |  |
| Restriction | NIL |  |  |  |
| - CXR – WUX – CXR |  |  |  |  |
|  | VVCR | ZSWX | CXRWUX0203A | ZGHA, ZSHC, ZSPD |
|  | ZSWX | VVCR | WUXCXR0302 | VVTS, VVBM |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: NIL |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – CAN – HAN |  |  |  |  |
|  | VVNB | ZGGG | HANCAN1102 | ZGNN, VHHH, VMMC |
|  | ZGGG | VVNB | CANHAN0211 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – CTU – HAN |  |  |  |  |
|  | VVNB | ZUUU | HANCTU1102 | ZUCK, ZLXY, ZPPP |
|  | ZUUU | VVNB | CTUHAN0211 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 REMARKS: NIL |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – HAK – HAN |  |  |  |  |
|  | VVNB | ZJHK | HANHAK1110 | VVDN |
|  | ZJHK | VVNB | HAKHAN1011 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – PEK – HAN |  |  |  |  |
|  | VVNB | ZBAA | HANPEK1118 | ZBTJ, ZBAD |
|  | ZBAA | VVNB | PEKHAN1811 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | ZGGG, ZSNJ, ZHCC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – PVG – HAN |  |  |  |  |
|  | VVNB | ZSPD | HANPVG1116 | ZSNB, ZSNJ, ZSHC |
|  | ZSPD | VVNB | PVGHAN3511 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | ZGGG, ZSNJ |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – SZX – HAN |  |  |  |  |
|  | VVNB | ZGSZ | HANSZX1115 | ZGGG, VHHH, VMMC |
|  | ZGSZ | VVNB | SZXHAN1511 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – CAN – SGN |  |  |  |  |
|  | VVTS | ZGGG | SGNCAN2502 | ZGNN, VHHH, VMMC |
|  | ZGGG | VVTS | CANSGN0225 | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VMMC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – PVG – SGN |  |  |  |  |
|  | VVTS | ZSPD | SGNPVG2517 | ZSNB, ZSNJ, ZSHC |
|  | ZSPD | VVTS | PVGSGN1725A | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – SZX – SGN |  |  |  |  |
|  | VVTS | ZGSZ | SGNSZX2515 | ZBAA, ZBTJ |
|  | ZGSZ | VVTS | SZXSGN1525 | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – PKX – SGN |  |  |  |  |
|  | VVTS | ZBAD | SGNPKX2517 | ZGGG, VHHH, VMMC |
|  | ZBAD | VVTS | PKXSGN1725 | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, ZGGG, ZSNJ, ZHCC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - CXR – CZX – CXR |  |  |  |  |
|  | VVCR | ZSCG | CXRCZX0211 | ZSCG, ZSNJ |
|  | ZSCG | VVCR | CZXCXR1102 | VVTS, VVBM |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, ZGGG, ZHHH |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – MFM – SGN |  |  |  |  |
|  | VVTS | VMMC | SGNMFM01 | VHHH, ZGSZ, ZGGG |
|  | VMMC | VVTS | MFMSGN01 | VVCR, VVCT, VVPQ |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – DNH – HAN |  |  |  |  |
|  | VVNB | ZLDH | HANDNH01 | ZLXY, ZLLL |
|  | ZLDH | VVNB | DNHHAN01 | VVVH, VVCI |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | ZUTF, ZLLL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – IZO – SGN |  |  |  |  |
|  | VVTS | RJOC | SGNIZO01 | RJBB, RJAA |
|  | RJOC | VVTS | IZOSGN01 | VVCR, VVPQ |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, ZGGG, VHHH, RCTP, RJFF, RJBB, RKPC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – FKS – SGN |  |  |  |  |
|  | VVTS | RJSF | SGNFKS01 | RJTT, RJAA |
|  | RJSF | VVTS | FKSSGN01 | VVCR, VVPQ |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, ZGGG, VHHH, RCTP, RJFF, RJBB |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – FKS – HAN |  |  |  |  |
|  | VVNB | RJSF | HANFKS01 | RJTT, RJAA |
|  | RJSF | VVNB | FKSHAN01 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | ZGNN, ZGGG, VHHH, RCTP, RJFF, RJBB |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - CXR – PVG – CXR |  |  |  |  |
|  | VVCR | ZSPD | CXRPVG01 | ZSNB, ZSNJ, ZSHC |
|  | ZSPD | VVCR | PVGCXR | VVTS, VVDN |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - CXR – TSN – CXR |  |  |  |  |
|  | VVCR | ZBTJ | CXRTSN01 | ZBAA, ZSJN |
|  | ZBTJ | VVCR | TSNCXR01 | VVTS, VVDN |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH, ZHHH, ZHCC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - CXR – NKG – CXR |  |  |  |  |
|  | VVCR | ZSNJ | CXRNKG01 | ZBTJ, ZHHH |
|  | ZSNJ | VVCR | NKGCXR01 | VVTS, VVDN |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VHHH, ZHHH, ZHCC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – HKG - HAN |  |  |  |  |
|  | VVNB | VHHH | HANHKG01 | ZGSZ, VMMC, ZGGG |
|  | VHHH | VVNB | HKGHAN01 | VVVH, VVCI |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – HKG - SGN |  |  |  |  |
|  | VVTS | VHHH | SGNHKG01 | RCTP, VMMC, ZGGG |
|  | VHHH | VVTS | HKGSGN01 | VVCR, VVPQ, VVCA |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |

| - SGN – KHH - SGN |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | VVTS | RCKH | SGNKHH01 | RCTP, VHHH, ZGGG |
|  | RCKH | VVTS | KHHSGN01 | VVCR, VVPQ, VVCA |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |

| - HAN – TAK – HAN |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | VVNB | RJOT | HANTAK01 | RJBB, RJOO, RJFF |
|  | RJOT | VVNB | TAKHAN01 | VVVH, VVCI |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, ZGGG, VHHH, RCTP, RJFF, RJBB |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – OKJ – HAN |  |  |  |  |
|  | VVNB | RJOB | HANOKJ01 | RJFF, RJBB, RJOO |
|  | RJOB | VVNB | OKJHAN01 | VVVH, VVCI |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, ZGGG, VHHH, RCTP, RJFF, RJBB |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - DAD – OKJ – DAD |  |  |  |  |
|  | VVDN | RJOB | DADOKJ01 | RJFF, RJBB, RJOO |
|  | RJOB | VVDN | OKJDAD01 | VVPB, VVBM, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | ZGGG, VHHH, RCTP, RJFF, RJBB |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HUI– ICN – HUI |  |  |  |  |
|  | VVPB | RKSI | HUIICN01 | RKSS, RKSM, RKTU |
|  | RKSI | VVPB | ICNHUI01 | VVDN, VVBM, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VHHH, RCTP, RKPC |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL300, FL340; FL400 (W-BOUND) Airway A593: THE EAST OF SADLI: Flights shall flight plan FL250, FL290, FL310 or FL390 (E-BOUND); FL240 FL280, FL300 or FL400 (W-BOUND) |  |  |  |
| - SGN – CTS – SGN |  |  |  |  |
|  | VVTS | RJCC | SGNCTS01 | RJSS, RJAA, RJTT |
|  | RJCC | VVTS | CTSSGN01 | VVCR, VVPQ, VVCA |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VVCR, VHHH, ZGGG, RCTP, RCKH, ZSPD, ROAH, RJBB |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway G86: Allocated flight levels: FL300, FL340; FL400 (W-BOUND) |  |  |  |

3.3.2.6 SOUTHEAST ASIA

| No | City pairs | Standard routes DB name | Preferred destination alternate airports |
| --- | --- | --- | --- |

| - DAD – BKK – DAD |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | VVDN | VTBS | DADBKK3501 | VTBD, VDTI, VLVT |
|  | VTBS | VVDN | BKKDAD1935 | VVPB, VVVH, VVNB |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – CGK – HAN |  |  |  |  |
|  | VVNB | WIII | HANCGK1107A | WIHH, WIPP, WSSS |
|  | WIII | VVNB | CGKHAN0711 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | WSSS |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – MNL – HAN |  |  |  |  |
|  | VVNB | RPLL | HANMNL1124A | RPLC, RPVM |
|  | RPLL | VVNB | MNLHAN0611 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VHHH |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV 5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – LPQ – HAN |  |  |  |  |
|  | VVNB | VLLB | HANLPQ1123 | VLVT, VTCC, VTCT |
|  | VLLB | VVNB | LPQHAN2311 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | NIL |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – KTI – HAN |  |  |  |  |
|  | VVNB | VDTI | HANKTI01 | VDSA, VVTS |
|  | VDTI | VVNB | KTIHAN01 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | NIL |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – SAI – HAN |  |  |  |  |
|  | VVNB | VDSA | HANSAI1105 | VDTI, VTBS |
|  | VDSA | VVNB | SAIHAN2311 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | NIL |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – VTE – HAN |  |  |  |  |
|  | VVNB | VLVT | HANVTE1102A | VTUD, VLLB, VTCC |
|  | VLVT | VVNB | VTEHAN0211 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – SIN – HAN |  |  |  |  |
|  | VVNB | WSSS | HANSIN | WMKJ, WMKK. WIDD |
|  | WSSS | VVNB | SINHAN1311 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVTS, WMKK |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - LPQ – SAI – LPQ |  |  |  |  |
|  | VLLB | VDSA | LPQSAI0235 | VDTI, VTBS |
|  | VDSA | VLLB | SAILPQ2323 | VLVT, VTCC, VTCT |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | NIL |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – BKK – SGN |  |  |  |  |
|  | VVTS | VTBS | SGNBKK2501 | VTBD, VDTI, VLVT |
|  | VTBS | VVTS | BKKSGN0125 | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – CGK – SGN |  |  |  |  |
|  | VVTS | WIII | SGNCGK2507 | WIHH, WIPP, WSSS |
|  | WIII | VVTS | CGKSGN0725A | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | WSSS |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – KUL – SGN |  |  |  |  |
|  | VVTS | WMKK | SGNKUL2514A | WMKJ, WMKP, WSSS |
|  | WMKK | VVTS | KULSGN1425 | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – MNL – SGN |  |  |  |  |
|  | VVTS | RPLL | SGNMNL2524 | RPLC, RPVM |
|  | RPLL | VVTS | MNLSGN2425 | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVCR |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV 5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – KTI – SGN |  |  |  |  |
|  | VVTS | VDTI | SGNKTI01 | VDSA, VVTS |
|  | VDTI | VVTS | KTISGN01 | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – SAI – SGN |  |  |  |  |
|  | VVTS | VDSA | SGNSAI2505 | VDTI, VTBS |
|  | VDSA | VVTS | SAISGN2325 | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – SIN – SGN |  |  |  |  |
|  | VVTS | WSSS | SGNSIN2502 | WMKJ, WMKK, WIDD |
|  | WSSS | VVTS | SINSGN0225A | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – DPS – SGN |  |  |  |  |
|  | VVTS | WADD | SGNDPS | WADL, WAAA |
|  | WADD | VVTS | DPSSGN | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – CEB – HAN |  |  |  |  |
|  | VVNB | RPVM | HANCEB01 | RPVI, RPMD, RPLL |
|  | RPVM | VVNB | CEBHAN01 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN, VVCR |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B, PBCS |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – KUL – HAN |  |  |  |  |
|  | VVNB | WMKK | HANKUL01 | WMKJ, WMKP, WSSS |
|  | WMKK | VVNB | KULHAN01 | VVVH, VVCI |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – BKK – HAN |  |  |  |  |
|  | VVNB | WMKK | HANKUL01 | VTBD, VDTI, VLVT |
|  | WMKK | VVNB | KULHAN01 | VVVH, VVCI |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| Restriction | NIL |  |  |  |
| - VTE – KTI - VTE |  |  |  |  |
|  | VLVT | VDTI | VTEKTI01 | VDSA, VVTS |
|  | VDTI | VLVT | KTIVTE01 | VTUD, VLLB, VTCC |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |

3.3.2.7 VIETNAM (DOMESTIC)

| No | City pairs | Standard routes DB name | Preferred destination alternate airports |
| --- | --- | --- | --- |

| - CXR – DAD – CXR |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | VVCR | VVDN | CXRDAD0235A | VVPB, VVVH, VVNB |
|  | VVDN | VVCR | DADCXR3520A | VVTS, VVBM |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: NIL |  |  |  |
| Restriction | NIL |  |  |  |

| - CXR – DLI – CXR |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | VVCR | VVDL | CXRDLI | VVBM, VVTX |
|  | VVDL | VVCR | DLICXR | VVTS, VVBM |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: NIL |  |  |  |
| Restriction | NIL |  |  |  |

| - CXR – HPH – CXR |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | VVCR | VVCI | CXRHPH0207 | VVNB, VVVH |
|  | VVCI | VVCR | HPHCXR0702 | VVTS, VVBM |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: NIL |  |  |  |
| Restriction | Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - CXR – HUI – CXR |  |  |  |  |
|  | VVCR | VVPB | CXRHUI2009 | VVDN, VVVH |
|  | VVPB | VVCR | HUICXR0920 | VVTS, VVBM |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: NIL |  |  |  |
| Restriction | NIL |  |  |  |
| - CXR – PQC – CXR |  |  |  |  |
|  | VVCR | VVPQ | CXRPQC2028 | VVCT, VVTS, VVCR |
|  | VVPQ | VVCR | PQCCXR2820 | VVTS, VVBM |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: NIL |  |  |  |
| Restriction | NIL |  |  |  |
| - CXR – TBB – CXR |  |  |  |  |
|  | VVCR | VVTH | CXRTBB0202 | VVBM, VVDL, VVCR |
|  | VVTH | VVCR | TBBCXR0202 | VVTS, VVBM |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: NIL |  |  |  |
| Restriction | NIL |  |  |  |
| - CXR – UIH – CXR |  |  |  |  |
|  | VVCR | VVPC | CXRUIH | VVBM, VVDL, VVCR |
|  | VVPC | VVCR | UIHCXR | VVTS, VVBM |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: NIL |  |  |  |
| Restriction | NIL |  |  |  |
| - DAD – BMV – DAD |  |  |  |  |
|  | VVDN | VVBM | DADBMV3509 | VVDL, VVCR, VVTS |
|  | VVBM | VVDN | BMVDAD0935 | VVPB, VVCR, VVTS |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: NIL |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) |  |  |  |
| - DAD – DIN – DAD |  |  |  |  |
|  | VVDN | VVDB | DADDIN | VVNB, VVCI, VVVH |
|  | VVDB | VVDN | DINDAD | VVPB, VVVH, VVNB |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: NIL |  |  |  |
| Restriction | NIL |  |  |  |
| - DAD – DLI – DAD |  |  |  |  |
|  | VVDN | VVDL | DADDLI3509 | VVBM, VVTX |
|  | VVDL | VVDN | DLIDAD0935 | VVPB, VVCR, VVTS |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: NIL |  |  |  |
| Restriction | NIL |  |  |  |
| - DAD – PQC – DAD |  |  |  |  |
|  | VVDN | VVPQ | DADPQC3528 | VVCT, VVTS, VVCR |
|  | VVPQ | VVDN | PQCDAD2835 | VVPB, VVCR, VVTS |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: NIL |  |  |  |
| Restriction | NIL |  |  |  |
| - DAD – PXU – DAD |  |  |  |  |
|  | VVDN | VVPK | DADPXU | VVBM, VVPB, VVDN |
|  | VVPK | VVDN | PXUDAD | VVPB, VVCR, VVTS |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - DAD – VCA – DAD |  |  |  |  |
|  | VVDN | VVCT | DADVCA3506 | VVTS, VVDL, VVCR |
|  | VVCT | VVDN | VCADAD0635 | VVPB, VVCR, VVTS |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - DAD – VDO – DAD |  |  |  |  |
|  | VVDN | VVVD | DADVDO | VVCI, VVNB, VVVH |
|  | VVVD | VVDN | VDODAD | VVPB, VVCR, VVTS |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - DAD – VII – DAD |  |  |  |  |
|  | VVDN | VVVH | DADVII3535 | VVTX, VVNB, VVDN |
|  | VVVH | VVDN | VIIDAD1735 | VVPB, VVCR, VVTS |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) |  |  |  |
| - HAN – BMV – HAN |  |  |  |  |
|  | VVNB | VVBM | HANBMV1109 | VVDL, VVCR, VVTS |
|  | VVBM | VVNB | BMVHAN0911 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - HAN – CXR – HAN |  |  |  |  |
|  | VVNB | VVCR | HANCXR1102 | VVTS, VVBM |
|  | VVCR | VVNB | CXRHAN0211 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 REMARKS: NIL |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - HAN – DAD – HAN |  |  |  |  |
|  | VVNB | VVDN | HANDAD1117 | VVPB, VVCR, VVTS |
|  | VVDN | VVNB | DADHAN3511 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - HAN – DIN – HAN |  |  |  |  |
|  | VVNB | VVDB | HANDIN1109 | VVNB, VVCI, VVVH |
|  | VVDB | VVNB | DINHAN0911 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – DLI – HAN |  |  |  |  |
|  | VVNB | VVDL | HANDLI1109 | VVBM, VVTX |
|  | VVDL | VVNB | DLIHAN0911 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - HAN – HUI – HAN |  |  |  |  |
|  | VVNB | VVPB | HANHUI1109 | VVDN, VVVH |
|  | VVPB | VVNB | HUIHAN0911 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - HAN – PQC – HAN |  |  |  |  |
|  | VVNB | VVPQ | HANPQC1110 | VVCT, VVTS, VVCR |
|  | VVPQ | VVNB | PQCHAN1011 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - HAN – PXU – HAN |  |  |  |  |
|  | VVNB | VVPK | HANPXU1109 | VVBM, VVPB, VVDN |
|  | VVPK | VVNB | PXUHAN0911 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - HAN – SGN – HAN |  |  |  |  |
|  | VVNB | VVTS | HANSGN1107 | VVCT, VVBM, VVCR |
|  | VVTS | VVNB | SGNHAN0711 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | VVDN |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - HAN – TBB – HAN |  |  |  |  |
|  | VVNB | VVTH | HANTBB1103 | VVBM, VVDL, VVCR |
|  | VVTH | VVNB | TBBHAN0311 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 REMARKS: NIL |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - HAN – UIH – HAN |  |  |  |  |
|  | VVNB | VVPC | HANUIH1115 | VVBM, VVDL, VVCR |
|  | VVPC | VVNB | UIHHAN3311 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - HAN – VCA – HAN |  |  |  |  |
|  | VVNB | VVCT | HANVCA1106 | VVTS, VVDL, VVCR |
|  | VVCT | VVNB | VCAHAN0611 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) |  |  |  |
| - HAN – VCL – HAN |  |  |  |  |
|  | VVNB | VVCA | HANVCL1114 | VVDN, VVPB, VVPC |
|  | VVCA | VVNB | VCLHAN1411 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 REMARKS: NIL |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - HAN – VDH – HAN |  |  |  |  |
|  | VVNB | VVDH | HANVDH1111 | VVVH, VVCI, VVDN |
|  | VVDH | VVNB | VDHHAN1111 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - HAN – VII – HAN |  |  |  |  |
|  | VVNB | VVVH | HANVII1117 | VVTX, VVNB, VVDN |
|  | VVVH | VVNB | VIIHAN1711 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 REMARKS: NIL |  |  |  |
| Restriction | Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - SGN – BMV – SGN |  |  |  |  |
|  | VVTS | VVBM | SGNBMV2509 | VVDL, VVCR, VVTS |
|  | VVBM | VVTS | BMVSGN0925 | VVCA, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – CXR – SGN |  |  |  |  |
|  | VVTS | VVCR | SGNCXR2502 | VVTS, VVBM |
|  | VVCR | VVTS | CXRSGN0225 | VVCI, VVVH |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – DAD – SGN |  |  |  |  |
|  | VVTS | VVDN | SGNDAD2535A | VVPB, VVVH, VVNB |
|  | VVDN | VVTS | DADSGN3525A | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - SGN – DIN – SGN |  |  |  |  |
|  | VVTS | VVDB | SGNDIN | VVNB, VVCI, VVVH |
|  | VVDB | VVTS | DINSGN | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - SGN – DLI – SGN |  |  |  |  |
|  | VVTS | VVDL | SGNDLI2509 | VVBM, VVTX |
|  | VVDL | VVTS | DLISGN0925 | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – HPH – SGN |  |  |  |  |
|  | VVTS | VVCI | SGNHPH2507A | VVNB, VVVH |
|  | VVCI | VVTS | HPHSGN0725A | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - SGN – HUI – SGN |  |  |  |  |
|  | VVTS | VVPB | SGNHUI2509 | VVDN, VVVH |
|  | VVPB | VVTS | HUISGN2725A | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – PQC – SGN |  |  |  |  |
|  | VVTS | VVPQ | SGNPQC2510 | VVCT, VVTS, VVCR |
|  | VVPQ | VVTS | PQCSGN1025 | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – PXU – SGN |  |  |  |  |
|  | VVTS | VVPK | SGNPXU2509 | VVBM, VVPB, VVDN |
|  | VVPK | VVTS | PXUSGN2725A | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) |  |  |  |
| - SGN – TBB – SGN |  |  |  |  |
|  | VVTS | VVTH | SGNTBB2503 | VVBM, VVDL, VVCR |
|  | VVTH | VVTS | TBBSGN0325 | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV10 (RNP10) |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – THD – SGN |  |  |  |  |
|  | VVTS | VVTX | SGNTHD2531A | VVVH, VVCI, VVNB |
|  | VVTX | VVTS | THDSGN3125A | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - SGN – UIH – SGN |  |  |  |  |
|  | VVTS | VVPC | SGNUIH2533A | VVBM, VVDL, VVCR |
|  | VVPC | VVTS | UIHSGN3325A | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) |  |  |  |
| - SGN – VCA – SGN |  |  |  |  |
|  | VVTS | VVCT | SGNVCA2506 | VVTS, VVDL, VVCR |
|  | VVCT | VVTS | VCASGN0625 | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – VCL – SGN |  |  |  |  |
|  | VVTS | VVCA | SGNVCL2514 | VVDN, VVPB, VVPC |
|  | VVCA | VVTS | VCLSGN1425A | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - SGN – VDH – SGN |  |  |  |  |
|  | VVTS | VVDH | SGNVDH2511 | VVVH, VVCI, VVDN |
|  | VVDH | VVTS | VDHSGN2925A | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - SGN – VDO – SGN |  |  |  |  |
|  | VVTS | VVVD | SGNVDO2503 | VVCI, VVNB, VVVH |
|  | VVVD | VVTS | VDOSGN0325A | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV2, RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - SGN – VII – SGN |  |  |  |  |
|  | VVTS | VVVH | SGNVII2517A | VVTX, VVNB, VVDN |
|  | VVVH | VVTS | VIISGN1725A | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | Required |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | RNAV5 |  |  |  |
| Datalink | CPDLC/ ADS-C, ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | Airway Q1: Allocated Flight Level: FL320; FL360 (S-BOUND) Airway Q2: Allocated Flight Level: FL310; FL350; FL390 (N-BOUND) |  |  |  |
| - SGN – CAH – SGN (FOR ATR) |  |  |  |  |
|  | VVTS | VVCM | SGNCAH2509 | VVCT, VVTS |
|  | VVCM | VVTS | CAHSGN0925 | VVCT, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | NIL |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – VCS – SGN (FOR ATR) |  |  |  |  |
|  | VVTS | VVCS | SGNVCS2511 | VVCT, VVTS |
|  | VVCS | VVTS | VCSSGN1125 | VVCA, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | NIL |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |
| - SGN – VKG – SGN (FOR ATR) |  |  |  |  |
|  | VVTS | VVRG | SGNVKG2526 | VVCT, VVTS |
|  | VVRG | VVTS | VKGSGN2625 | VVCA, VVPQ, VVCR |
| Area Operation | MID/ASIA |  |  |  |
| Preferred Enroute Alternate Airport | NIL |  |  |  |
| RVSM | NIL |  |  |  |
| EDTO | Not-required |  |  |  |
| PBN | NIL |  |  |  |
| Datalink | ADS-B |  |  |  |
| Flight level | MIN – MAX: GND – FL410 Remarks: Nil |  |  |  |
| Restriction | NIL |  |  |  |

# 3.4 POSITION FIXING, REPORTS AND WAYPOINTS

Except where operational considerations dictate otherwise, reporting points should be located at intervals of 5 degrees of latitude or longitude, depending if the route is predominantly north-south or east-west.

ACFT traversing 10 degrees of latitude or longitude in 1 HR 20 MIN or less should normally be required to report only at 10-degree intervals. Slower ACFT should normally be required to report at 5-degree intervals.

Normal position reporting procedures should be continued at all times, regardless of any action taken to initiate or acknowledge a traffic information broadcast.

- In Australian FIR a position report must be made on the next CTA/FIA frequency 15 minutes prior to leaving the airspace in which TIBA procedures apply to obtain onwards clearance.

- Within the Fukuoka (excluding the area west of a line 100 NM seaward from the eastern coastline of Japan), Incheon, Manila (East of the Philippines) and Taipei FlRs, flights shall provide position reports.

The position of the ACFT shall be transmitted in reference to a reporting point name, name-code designator or, if not named:

- for flights operating in a predominantly east-west direction:

- latitude in degrees and minutes; and

- longitude in degrees only;

- for flights operating in predominantly north-south direction:

- latitude in degrees only; and

- longitude in degrees and minutes.

The time at which the ACFT is over the reporting point shall be transmitted in four digits, giving both the hour and the minutes. Estimated time over next position shall be expressed in four digits.

# 3.5 ENROUTE CHARTS

Currently, Vietnam Airlines uses LIDO Supplier for Charting.

For details Enroute charts, please refer Lido mPilot (EFB1), eRoute Manual (EFB2, EFB3).

Figure 3-1 Example of En-Route map in mPilot (EFB1)

Figure 3-2 Example of En-Route map in EFB2

Figure 3-3 Example of En-Route map in EFB3

For more instruction details in Enroute charts, please refer to “Help” button in LIDO mPilot, section Enroute Map View

Airway Q9, W27 and W28 within Vietnam airspace (Which are available in the Navigation Database for A350 and B787 aircraft) as the following information: